SECTION 11 6500

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Pole Vault Box.

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Manufacturers and product selections named are provided to establish the minimum standard and shall comply with the safety standards as outlined below (2.02B). Equipment shall be as supplied by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

www.sportsedge.com

info@sportsedge.com

B. Substitutions:  No Substitutions.

2.02	TRACK AND FIELD EQUIPMENT: Pole Vault Box with Cover and Safety Collar

1. Vault Box: Model SE507 SKYDEX™ Soft Pole Vaulter’s Box to consist of a HMPWE cover, steel support, Skydex cushioning material, and fiberglass tray. The HMPWE cover is 1/8” thick and white. The steel support backing plate is 1/4” thick. The cushioning material is made up of two layers of Skydex material. The fiber glass tray is 1/8” thick with a white gel coated interior.

2. Vault Box Lid: Model SE50701, steel

3. Vault Box Safety Collar Model #SE719 (required)

PART 3: EXECUTION

3.03	INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --