SECTION 33 4460

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

1.05	SYSTEM DESCRIPTION:

System to be of one-half meter in-line catch basin of polymer concrete with integral cast iron 		frames,	removable cast iron grates, grate locking devices, and accessories. Assembly: 			PolyDrain 611 Large Catch Basin.

PART 2: PRODUCTS

2.01 	PREFABRICATED CHANNEL DRAIN CATCH BASIN

- BASE: Shall be PolyDrain 611 Large Catch Basin as manufactured by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

FX: 704-528-0179

info@sportsedge.com

www.sportsedge.com

2.02	COMPONENTS

A.   Catch basin, fabricated of polymer concrete for use with precast trench drains or for stand-alone application, meeting the following requirements:

a. Consisting of three sections with the bottom section having a solid floor.

b. Length: 19.7 in (500mm) x 12.6” (324mm)

c. Overall Depth: 48.3” (1227mm)

d.  Preformed cutouts for varying depth channels.

e. Preformed 6” cutouts for insertion of drainage pipe.

B. Frame & Grate:

a. 2603B Cast Iron Frame factory adhered to catch and 2604B Inlay Cast Iron Grate

C. Accessories:

a. Four (4) corner bolts that secure grate to frame

b. Optional: 2605 Galvanized Steel or 2608 Stainless Steel removable trash bucket

## PART 3: EXECUTION

3.01 INSTALLATION / QUALITY ASSURANCE

A. Install per the manufacturer’s instructions and recommendations.

END OF SECTION