SECTION 11 6813

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 11 6500 - 	Athletic & Recreational Equipment

Section 12 9300 - 	Site Furnishings

Section 31 2300 - 	Excavation and Fill

Section 33 3000 - 	Site Improvements

Section 32 1820 - 	Athletic & Sports Surfaces

Section 32 1826 - 	Synthetic Grass Surfacing

Section 32 1840 - 	Playground Protective Surfaces

Section 32 9200 -	Turf and Grasses

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of rubber border. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- Rubber Playground Corner Guard System; Rubber Curb with chamfered edge, Overall  6.5”W x 4”T x 96”L, shall be manufactured of 100% recycled rubber, Model # SEFLEXPCG.

### PART 2 PRODUCTS

2.01 	Rubber Playground Curbing;

A. Base: Model #SEFLEXPCG or by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

- COMPONENTS:

1. Rubber Playground Corner Guard Model #SEFLEXPCG meeting the following 	criteria as the minimum standard:

a. Rubber Curb:

i. 6.5”W x 4"H x 96"L, countersunk holes for placing anchoring bolts 	into concrete curbing

ii. Manufactured of 100% recycled rubber
	iii. Manufactured of 90% post-consumer content

b. Accessories:

i. Betaseal Adhesive, 10.5 oz., part # SEFLEXBST

ii. UV Topcoat - 4 Liter, part # SEFLEXTC

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. Install as recommended by the manufacturer, and as indicated on the drawings. The installing contractor shall ensure that the base is compacted and level to correct elevation.

END OF SECTION