SECTION 33 4460

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

1.05	SYSTEM DESCRIPTION:

System to be of one meter (1m) long non-sloping drain channels of polymer concrete with 		removable grates, grate locking devices, and accessories. Assembly: Pro-S with #338			Thermoplastic Grate.

PART 2: PRODUCTS

2.01 	PREFABRICATED NON-SLOPING CHANNEL DRAIN

- BASE: Shall be Pro-S with #338 Thermoplastic Grates and grate locking devices as manufactured by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

FX: 704-528-0179

info@sportsedge.com

www.sportsedge.com

2.02	COMPONENTS

A.   Channels: non-sloping channels fabricated of polymer concrete, 6 inches wide, 4-inch ID and with radius bottom meeting the following requirements:

a. Length: 1 meter (39.2 in).

b. Depth: 8.8” (222mm)

c. Anchoring ribs: Full Length

d. Grate Locking Slots: blind, vibration dampening, thermoplastic.

e. Interlocking Ends

B. Grates: Manufactured Thermoplastic.

a. Model: #336 Thermoplastic Slotted

b. Width: 124mm (4.9”)

c. Length: 0.5 meter (19.6”)

d. Open Area: 0.118 ft2 per linear foot

e. Load rating: 75psi (pedestrian)

C. Accessories:

a. Grate Locking devices.

## PART 3: EXECUTION

3.01 INSTALLATION / QUALITY ASSURANCE

A. Install per the manufacturer’s instructions and recommendations, or by a factory trained 	and authorized installer.