Section 33 4460

PART 1: GENERAL

1.01 	RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification 	sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

Section 33 4000 - Storm Drainage Utilities

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

A. SportsEdge ProTect Channel Drain System of one meter (1m) long drain channels manufactured of Polymer Concrete with channel lip removed on one side. The system shall have a three-meter length protective trim edge guard at the surface. Polymer concrete channel edge shall not be exposed in the finished installation on the field side. System to include removable ADA and heel proof polypropylene grates, grate locking devices, and accessories. Assembly number 2192M336-(NAT or SYN).

PART 2: PRODUCTS

2.01 PREFABRICATED CHANNEL DRAIN ACCEPTABLE MANUFACTURERS SURFACE 	DRAIN: Base; as manufactured by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

F: 704-528-0179

info@sportsedge.com

www.sportsedge.com

## 2.02    SURFACE DRAINAGE SYSTEM

A. Channel: ProTect Channel # 2192M336-(NAT or SYN) fabricated of polymer 			concrete, 6 inches wide, 4-inch ID and 8.75” deep minimum with radius non-sloping 		bottom having the following attributes:

1. Length: 1 meter (39.2 in).

2. Depth: 8.75" minimum O.D.

3. Anchoring ribs: full length.

4. Grate locking slots: blind, vibration dampening, thermoplastic.

5. Interlocking ends.

6. Channel grate lip removed (one side)

B. Grates: Fabricated of polypropylene, part #2338

1. Longitudinally slotted, must meet heel proof (< 0.3125”) and ADA (< 0.5”) 		requirements.	6mm (0.236”) drainage openings

2. Load Rating: 150 psi minimum

3. Open Area: 27.47 in² Minimum

4. Color: Black, UV stabilized.

5. Length: 1/2 meter (19.7")

C.   Protective Edge Guard shall be Model # 2790CAP and have the following 			attributes:

1. Black Thermoplastic Elastomer (TPE), UV stabilized.

2. Length: 1.625” H x 1.25” W x 118.2” (3 Meter)

3. 80 - 85 shore A durometer.

D.   Accessories:

1. Grate Locking devices.

## PART 3: EXECUTION

3.03 INSTALLATION / QUALITY ASSURANCE

A. Install per the manufacturer’s instructions and as indicated on the plans;

Specifier note: For natural grass applications select Part #2192M336-NAT.

For synthetic turf select Part #2192M336-SYN. Components are identical,

but the installation cross section varies.

END OF SECTION