SECTION 11 6833

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

A. Football goals manufactured of Aluminum with an 8’ offset, and 20’ length uprights. Assembly number SEF305R Rotating.

### PART 2 PRODUCTS

2.01 FOOTBALL GOAL POST

A. Base: Model #SEF305R Sleeve Type, Rotating as manufactured by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

FX: 704-528-0179

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Football goals shall be manufactured of all aluminum with an 8’ (96”) offset; powder coated yellow RAL1016 and must meet the following criteria as a minimum.

1. Gooseneck: fabricated of .280” thickness 6” SCH40 aluminum pipe, 6.625” OD minimum.

a. Rotatable up to 360 degrees

b. Internal Shaft:

    - shall be solid aluminum

    - 47.5” min. length to provide maximum stability

c. Internal Bearing System:

    - shall have 6” minimum diameter bearings

    - shall have two bearing points 31” apart to provide maximum stability.

    - shall not require lubrication

    - shall be made of dissimilar metals for corrosion resistance

d. Rotation point shall be above finished grade.

e. Shall have two keyed locking points to ensure 180 degree rotation for squareness to the end line.

f. Gooseneck unit must be capable of re-calibration for precise alignment of the crossbar to the end line.

2. Crossbar: fabricated of .25” thickness 6” SCH40 aluminum pipe, (6.625 in)

OD minimum.

- Length: High School: 23’- 4” or College: 18’ 6” (specifier select)

between uprights.

- Uprights: fabricated of aluminum tube, 4.0” OD minimum (0.125”

wall thickness), Length: 20’-0”  or 30’-0” (specifier select)

4. Installation package consisting of the following components:

- Ground Sleeve (REQUIRED):  8” SCH40 steel pipe, 5’ long, 10”x10”.125” Steel Plate.

- Access Kit: fabricated of aluminum. Include cut-outs: fabricated of pressure treated plywood and artificial turf for grass field. Synthetic turf fields to have plywood cutouts wrapped on the job to match carpet.

5. Accessories:

- 48” Handle for rotating the goal post when required.

- 4 ea. - Directional wind flags.

- Assembly bolts, nuts and spring (set) pins.

- Color matched spray / touch-up paint.

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended by manufacturer, and as indicated on the drawings.

END OF SECTION