SECTION 11 6833

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 11 6500 - 	Athletic & Recreational Equipment

Section 12 9300 - 	Site Furnishings

Section 31 2300 - 	Excavation and Fill

Section 33 3000 - 	Site Improvements

Section 32 1820 - 	Athletic & Sports Surfaces

Section 32 1826 - 	Synthetic Grass Surfacing

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of rubber border. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- Flexible Rubber Baseball Divider Curb with a flexible rubber insert for Synthetic Turf to clay/grass applications; manufactured of 100% recycled rubber, assembly number SEFLEXBDC.

### PART 2 PRODUCTS

2.01 	FlexEdge® Rubber Baseball Divider Curb for Synthetic Turf

A. Base: Model #SEFLEXBDC by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

FX: 704-528-0179

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS: Rubber Baseball Divider Curb for synthetic turf, shall be manufactured of 100% recycled rubber, and must meet the following criteria as the minimum standard:

1. Baseball Divider Curb:

a. 3" High x 5” Wide, 0.75” recess for flexible rubber insert

b. Chamfered inside edge

c. Manufactured of 100% recycled rubber
d. Manufactured of 90% post-consumer content

2. Flexible Divider:

a. Polymer SBR/BR

b. 2.25” overall height, 1.5" height above rubber curb

c. Woven nylon polyester chord

d. Hardness (Shore A) 60

e. UV inhibitors

3. Tubing:

a. 0.75” x 0.75” square aluminum tubing

b. Interlocking doweled ends

c. Pre-radiused as required

4. Accessories:

- 0.5” x 18” Anchor Pins, Steel w/ welded cap, part # SEFLEXA18

- Bata Seal Adhesive, 11 oz., part # FLEXBST

- UV Top Coat - 4 Liter, part # SEFLEXTC

- D. 10” spikes

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. Install as recommended by the manufacturer, and as indicated on the drawings. The installing contractor shall ensure that the base is compacted and level to correct elevation.

END OF SECTION