SECTION 02880

PART 1	 - GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. 8” Adjustable Take Off Board System

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2	 - PRODUCTS

2.01	MANUFACTURERS

A.	Manufacturers and product selections named are provided to establish the minimum standard and shall comply with NFHS and NCAA requirements.

Equipment shall be as supplied by:

SportsEdge®, P.O. Box 837, 259 Murdock Rd. Troutman, NC 28166

Phone: 800-334-6057		Fax: 704-528-0179

Email: info@sportsedge.com	www.sportsedge.com

B.	Substitutions:  Requests for substitutions will be considered in accordance with Section 01630.

2.2	LONG JUMP & TRIPLE JUMP TAKE-OFF BOARD SYSTEMS

A.	Long Jump & Triple Jump Take-Off Board Systems:  Model No. SE444 Take-Off Board System.

1.	Aluminum Tray:  Constructed of 3/16” thick aluminum, formed and welded.

- Aluminum Base Board: Constructed of 1/8” aluminum sheet (3003), formed to shape. All adjustment screws are stainless steel. The base board can be adjusted for level and height, using six adjustment screws, without removing the base board from the tray. There are four threaded fasteners for attaching the take-off board.

- Wood Take-Off Board: Constructed of ¾” A/B marine grade plywood with a white outdoor enamel coating. There are four counter-sunk holes for mounting to the base board. The take-off board is finished on both sides so that it can be reversible.

PART 3	 - EXECUTION

3.03 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --