# SECTION 02880

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast In Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

A.  Combination football / soccer goals, steel 8’H x 24’W regulation soccer goal, 23’4"W regulation football goal with 15 Ft. uprights.

### PART 2 PRODUCTS

2.01 COMBINATION SOCCER / FOOTBALL GOALS

A. Base: Shall be Model #SE720 as supplied by:

SportsEdge®

P.O. Box 837

Troutman, NC  28166

PH: 800-334-6057

Fax: 704-528-0179

Email: info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Model #SE720 combination football / soccer goal shall be manufactured of aluminum and have a white powder coated finish.  Goal posts should meet the following criteria as a minimum standard.

- 8’H x 24’W x 3’D x 8’B soccer goal, 10’H x 23’-4”W football goal with 15’H Uprights

- Soccer Front Posts & crossbar: 4.375” round x 0.115” wall

- Soccer Backdrop: 1.25” round x 0.125” wall, 36” depth

- Football Crossbar: 2” square  x 0.125” wall

- Football Uprights: detachable 2” round x 0.125” wall, 15’ Height

- Powder coated white finish with safe, smooth radius corners.

- Includes: 4.0mm Soccer Net (Orange, Blue, Yellow or Red).

- Ground Sleeves:  24.25” Depth with caps

- Net: 3mm, white net (0050AW), Kwik Lock® Net Clips

- Optional: Soccer Goal Post Pads:

  - ¾” closed cell pads with hoop and loop strap attachment

  - 18 oz. vinyl

  - Color: White

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended by manufacturer, and as indicated on the drawings.

END OF SECTION