SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Portable Discus Cage and Platform, High School

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Manufacturers and product selections named are provided to establish the minimum standard and shall comply with the requirements in NFHS rule book.

Equipment shall be as supplied by:

SportsEdge, P.O. Box 837  259 Murdock Rd. Troutman, NC 28166

Telephone: 800-334-6057  Fax: 704-528-0179  Email: info@sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with requirements in NFHS rule book.

B. Portable Discus Cage and Platform:

1. CAGE: Model # SE732220, High School

a. Upright poles; (6) 14’ tall net poles to be 2½” schedule 40 aluminum pipe with 33” rolled offsets and mounted on weighted base assemblies.

b. Bases; fitted with two 8” ball bearing wheels, which allow pole assemblies to be rocked back and easily moved. Once in position, four 5 gallon jugs to be filled with water or sand, secure net poles in place.

c. Nets; 14’ tall by 55’ main net is to be constructed of #504 weather-treated nylon netting with a breaking strength of over 200 lbs. Weather treated nylon nets and vinyl coated steel net support cables to reduce net sag, ensuring maximum net height. Nets are to be secured to bases by replaceable shock absorbing bungee cords.

i. Optional: 8’ outer barrier nets.

2. PLATFORM: Model # SE3710 Portable Discus Platform; to be 12'x12'x2" wood platform with a 2.5m diameter aluminum circle.

a. Platform to be constructed of 1/2" marine grade plywood reinforced with pressure treated 2"x4" and 2"x6" lumber.

b. Platform is painted in gray marine coat finish, with 2” wide white sector lines set at 34.92 degrees.

c. Platform to break down into four 8' x 4' sections, and four 8' length, 2" x 6" stringers.

d. Aluminum discus circle is SportsEdge® model SE370.

PART 3: EXECUTION

3.01 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

END OF SECTION –