SECTION 02880

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02542 - Synthetic Grass & Turf

Section 02870 – Site Furnishings

Section 03300 – Cast In Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- Access Frame and Soccer Goal Safety Anchor System to securely anchor the ground bar of the soccer goal.

### PART 2 PRODUCTS

2.01 SOCCER GOAL SAFETY ANCHOR SYSTEM

A. BASE: Shall be as supplied by:

SportsEdge®

P.O. Box 837, 259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057   Fax: 704-528-0179   www.sportsedge.com

B. COMPONENTS

SOCCER GOAL SAFETY ANCHOR SYSTEM: Model SE790 aluminum Frame with nylon straps, pressure treated plywood covers and aluminum anchors to secure the ground bar of the soccer goal.

1. Access Frame & Covers:

- Fabricated of 0.125” thickness, aluminum, 17.75” square, 11.5”H, with four anchor bolts.

- Filler Plugs / Covers: fabricated of pressure treated plywood. Synthetic turf to be field applied by synthetic turf contractor.

2. Anchoring/Ratchet System:

- Aluminum hook, 7.5”H, 12.5”L, 2.25”W,  1 per box

- Nylon Straps, 1.9” width

- Aluminum internal parts and stainless steel hardware.

- Fully removable apparatus for future replacement as required.

4. Installation package consisting of the following components:

- Access Frame

- Nylon anchor straps and ratchet system with hooks.

- Installation / assembly bolts, nuts.

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended by manufacturer, and as indicated on the drawings.

END OF SECTION