SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Discus Cage	, Aluminum, High School

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Substitutions: Requests for substitutions will be considered in accordance with Section 01630.

B. Manufacturers and product selections named are provided to establish the minimum standard and shall comply with the requirements in NFHS rule book. Equipment shall be as supplied by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with requirements in NFHS rule book.

B. Discus Cage Model # SE8010

- Ground Sleeves: Consists of six (6) ground sleeves, constructed of 3” Sch40 PVC pipe, 30” long.

- Net Poles: Consists of six (6) net poles, constructed of 2½” Sch40 aluminum pipe. The net poles will be off-set and stand at least 14’ tall when installed in the ground sleeves. The net poles will have welded on rope tie-off cleats. The top of the net poles will be capped. The net poles will have holes for installing hardware to hang the net from.

- Hoisting Rope: The rope used to hang the net from the net poles will be 1/4” braided polyester, 25’ long.

- Net: Constructed of 100% white nylon 6 yarn solution dyed black, 1 7/8”sq mesh (3 ¾” stretch mesh), raschel knotless netting, UV stabilized and heat-set, 180lb break strength, rope bound, 14’ x 56’

- Net Support Cable: Constructed of 1/8”-3/16”, 7x19, PVC coated, galvanized steel cable. The top of the net will be supported by the net support cable along the entire length of the net.

- Flip Arms: Consists of two (2) flip arms constructed of 3/8” x 2” steel and 32” long, with a powder coat finish. The flip arms will be attached to the two front net poles at ground level and to the front bottom corners of the net to keep the net secured.

- Ground Stakes: Constructed of plastic and 9” long. The ground stakes will be used to secure the net to the ground at the rear poles and between each pole.

- Hardware: All hardware is to be zinc plated.

- Warning Signs: Consists of three (3) warning signs constructed of polyethylene board 10” x 14”. The warning signs will be fastened to the outside of the net, at eye level, facing outward, at the front sides of the discus cage and at the rear of the discus cage.

a. Optional: #SE80102 Barrier Net. Net: Consists of two nets constructed of 100% white nylon 6 yarn solution dyed black, 1 7/8”sq mesh (3 ¾” stretch mesh), raschel knotless netting, UV stabilized and heat-set, 180lb break strength, rope bound, 8’ x 64’-7”.

PART 3: EXECUTION

3.03 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --