SECTION 11 6500

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. 30’ Ball Stop System with ground sleeves.

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS/SUPPLIERS

A. Manufacturers/Suppliers and product selections named are provided to establish the minimum standard and shall comply with the following by:

SportsEdge

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with the following requirements.

B. Ball Stop System with Pulley System:

1. Please select: Model #SEBS30 with Mill Finished Poles or #SEBS30P with Powder Coated Poles:

a. Poles: 35’-4” length (30’-6” above grade), 6.0” O.D. x 0.25” thick wall 6061-T6 Aluminum.

b. Netting: #M-500 Black (HTPP) w/ extra UV stabilizers; 1.75” Mesh. Mesh Break: 180 lbf.

c. Cable: 3/16” vinyl coated net support cable at the top and bottom to reduce sag.

d. All hardware to be Stainless Steel or Marine Grade.

e. Includes Rope Pulley System with Cleat Tie-off for lowering/raising net.

f. Ground sleeves: 5’ Length / 6” Sch. 40 Aluminum, Alloy 6061-T6 with Stainless Steel Stop-Bolt.

g. Ground Sleeve and Pole Caps included.

PART 3: EXECUTION

3.03	INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --