Section 33 4460 Surface Drains

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

Section 33 4000 - Storm Drainage Utilities

SYSTEM DESCRIPTION:

System to be of one meter (1m) long Presloped drain channels of polymer concrete with removable grates, grate locking devices, and accessories. Assembly: Presloped Drain System.

PART 2: PRODUCTS

2.01 PRE-FABRICATED PRESLOPED CHANNEL DRAIN

ACCEPTABLE MANUFACTURERS

Surface drain:

- BASE:

- Shall be SportsEdge® / ABT channel drain with grates and grate locking devices as manufactured by:

SportsEdge®

P.O. Box 837 259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

## B. COMPONENTS

1.   Channels: built in 0.6% slope with non-sloped channels as required to extend trench runs, fabricated of polymer concrete, 6 inches wide, 4 inch ID and with radius bottom meeting the following requirements:

a. Length: 1 meter (39.2 in).

b. Depth: 5.1’ Minimum to 12.2” Deep Maximum

c. Anchoring ribs: Full Length

d. Grate Locking Slots: blind, vibration dampening, thermoplastic.

e. Interlocking Ends

2. Grates: Fabricated of Thermoplastic Polymer.

- #2336 Herringbone Pattern must meet:

- Heel proof (0.25”<) and

- ADA (0.5”<) requirements.

- .25” (6.2mm) drainage openings.

- Color: Black

- Length: 1/2 meter (19.7")

- UV stabilizers

3.  Catch basins: Model #2900 fabricated of polymer concrete

a. Width:  6" OD, 4" ID

b. Depth:  22.1"

c. Length: 1/2 meter (19.7")

d. Grate: #2336.

4. Accessories:

a. Grate Locking devices.

## PART 3: EXECUTION

3.03 INSTALLATION / QUALITY ASSURANCE

1. Install per the manufacturer’s instructions and recommendations, or by a factory trained and authorized installer.