# SECTION 11 6833

### PART 1 - GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

A.  Soccer Goals, all aluminum construction, meeting NCAA and NFSHSA specifications. Assembly number SE700R

PART 2 - PRODUCTS

2.02 SOCCER GOALS

A. BASE: Shall be Model #SE700R by:

SportsEdge

P.O. Box 837

259 Murdock Road

Troutman, NC  28166

PH: 800-334-6057

FX: 704-528-0179

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Model #SE700R Soccer Goals meeting NCAA and NFHS specifications; shall be all aluminum construction with White powder coated finish and must meet the following criteria as a minimum standard:

1. Frame: 8’H x 24’W x 4’B x 10’D.

  - White powder coated finish

- Backstays:

  - 2” O.D. Aluminum Backstays, 0.125” Wall thickness

  - White powder coated finish

- Ground Bar:

  - 2” O.D. Aluminum, 0.125” Wall thickness

  - White powder coated finish

  - Must be adjustable & moveable to accommodate football gooseneck

- Net Attachments:

  - Crossbar & Uprights: Twist in nylon net clips

  - Backstays & Ground bar” Hoop & Loop strap

5. Nets: #SE755 3mm HTTP Nets, 4.0” square mesh.

6. Portable Ground Anchors:

a. Natural Grass Field Applications: #SE752 16” Steel Spiral Anchor w/ Lanyards & quick clip attachment (Set of 4)

b. Synthetic Turf Field Applications: #SE753 (Set of 4) Heavy Duty Nylon Sand Bags (Holds 40 lbs.)

7. Wheel Kit: #SE751 rear mounted backstay wheel attachment (set of 4), 10” flat free polyurethane Marathon® tire with nylon bearing.

### PART 3 - EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended by manufacturer, and as indicated on the drawings.

END OF SECTION