# SECTION 11 6600	Athletic Equipment - Field

### PART 1	 GENERAL

1.01	 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - DESCRIPTION

A. Football goal post pads, 6.0’ height, Velcro closure.

### PART 2	 PRODUCTS

2.01 FOOTBALL GOAL POST PADS

Base: Shall be Model #SEF302 by:

SportsEdge®

P.O. Box 837, 259 Murdock Road

Troutman, NC  28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

1. COMPONENTS

a. 13” minimum thickness split cylindrical urethane foam core

b. Vinyl laminated polyester fabric, 18 oz. / square yard minimum weight.

c. Minimum height of (6.0’) 72”.

d. Full length Velcro closure strip.

*Note: specifiers / owner to select from the following standard colors. 				  		Gold, Black, Red, Forest Green, Kelly Green, Brown, Maroon, Orange, Gray,

Royal, Navy, Purple, Tan, White, Sky Blue and Teal.

2. ACCESSORIES:

a. Custom colors, logo, and lettering are optional.

### PART 3 EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A.  All athletic equipment shall be installed as recommended with manufacturer's written directions, and as indicated on the drawings.

END OF SECTION