SECTION 11 6500

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 11 6500 - 	Athletic & Recreational Equipment

Section 12 9300 - 	Site Furnishings

Section 31 2300 - 	Excavation and Fill

Section 33 3000 - 	Site Improvements

Section 32 1820 - 	Athletic & Sports Surfaces

Section 32 1826 - 	Synthetic Grass Surfacing

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of rubber border. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- Long Jump Pit 9’-0” I.D. x 26’-0” I.D. comprised of a Rubber Curb Border System with a built in ledge for aluminum pit cover set; shall be manufactured of 100% recycled rubber, assembly number SEFLEXLJC2.

### PART 2 PRODUCTS

2.01 	Long Jump Pit Rubber Curb and Border System

A. BASE: Model #SEFLEXLJC2 by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS: Long Jump Pit Rubber Curb and Border System, shall be manufactured of 100% recycled rubber, and must meet the following criteria as the minimum standard:

1. Rubber Long Jump Pit Curb and Pre-fabricated Corners:

a. 7.75”W x 10"H

b. Ledge for Aluminum Pit Covers, 2”W x 2.5”D

c. Manufactured of 100% recycled rubber
d. Manufactured of 90% post-consumer content

2. Aluminum Pit Covers #SE730148:

a. Recessed top surface: 0.5” (13mm)

b. Retractable handles

c. Frame: All Aluminum, 2.375”D

d. Cover Length: 9’ - 3 1/2”

4. Accessories:

- 18” Anchor Pins, Steel w/ welded cap, part # SEFLEXA18

- Betaseal Adhesive, 10.5 oz., part # SEFLEXBST

- UV Top Coat - 4 Liter, part # SEFLEXTC

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. Install as recommended by the manufacturer, and as indicated on the drawings. The installing contractor shall ensure that the base is compacted and level to correct elevation.

END OF SECTION