SECTION 11 6500

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 11 6500 - 	Athletic & Recreational Equipment

Section 12 9300 - 	Site Furnishings

Section 31 2300 - 	Excavation and Fill

Section 33 3000 - 	Site Improvements

Section 32 1820 - 	Athletic & Sports Surfaces

Section 32 1826 - 	Synthetic Grass Surfacing

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of rubber border. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- Long Jump Pit Rubber Curb and Form System manufactured of 100% recycled rubber, assembly number SEFLEXLJC3.

### PART 2 PRODUCTS

2.01 	Long Jump Pit System

A. BASE: Model #SEFLEXLJC3 by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS: Long Jump Pit Rubber Curb & Form System, shall be manufactured of 100% recycled rubber, and must meet the following criteria as the minimum standard:

1. Rubber Long Jump Pit Curbs & Forms:

a. 7.75”W x 10"H

b. Manufactured of 100% recycled rubber
c. Manufactured of 90% post-consumer content

d. Pre-fabricated Corners

e. 2.5”D x 2”W Built-In Ledge for Pit Cover Set

f. 1.625”D x 2”W Built-In Ledge for Sand Catcher Grate & Mat

2. Pit Cover Set

a. Recessed top surface: 0.5” (13mm)

b. Retractable handles

c. Frame: All Aluminum, 2.375” - 2.5”D

d. Panel Length: 9’ - 3 ½”

3. Sand catcher components:

a. Sand catchers: Poured in place concrete floor, 2500 PSI Min.

b. Mat support: Galvanized steel or aluminum mesh bar grate

c. Mat: Resilient rubber mesh style, UV resistant.

4. Accessories:

- 18” Anchor Pins, Steel w/ welded cap, part # SEFLEXA18

- Bata Seal Adhesive, 11 oz., part # FLEXBST

- UV Top Coat - 4 Liter, part # SEFLEXTC

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. Install as recommended by the manufacturer, and as indicated on the drawings. The installing contractor shall ensure that the base is compacted and level to correct elevation.

END OF SECTION