Section 33 4460 Precast Track Perimeter Channel Drain

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

Section 33 4000 - Storm Drainage Utilities

SYSTEM DESCRIPTION

System to be SportsEdge ProTect Channel Drain System of one meter (1m) long drain channels manufactured of Polymer Concrete with channel lip removed on one side. The system shall have a three meter length protective trim edge guard at the surface. Polymer concrete channel edge shall not be exposed in the finished installation on the field side. System to include removable ADA / heel proof grates, grate locking devices, and accessories.

Assembly number SE2192M/336.

PART 2: PRODUCTS

2.01 PRE-FABRICATED CHANNEL DRAIN

ACCEPTABLE MANUFACTURERS

SURFACE DRAIN: Base; as manufactured by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

F: 704-528-0179

info@sportsedge.com

www.sportsedge.com

## B. COMPONENTS

1. Channel: ProTect Channel # 2192M fabricated of polymer concrete, 6 inches wide, 4 inch ID and 8.75” deep minimum with radius non-sloping bottom having the following attributes:

a. Length: 1 meter (39.2 in).

b. Depth: 8.75" minimum O.D.

c. Anchoring ribs: full length.

d. Grate locking slots: blind, vibration dampening, thermoplastic.

e. Interlocking ends.

f. Channel grate lip removed (one side)

2. Grates: Fabricated of thermoplastic polymer.

- #2336 Herringbone Pattern: Meeting Heel proof (0.25”<) and ADA (0.5”<) requirements.

- 6.2 mm drainage openings.

- Color: Black

- Length: 1/2 meter (19.7")

- UV stabilizers

3.   Protective Edge Guard shall be Model # 2790CAP and have the following attributes:

a. Black Thermoplastic Elastomer (TPE)

b. UV stable

b. 1.625” H x 1.25” W x 118.2” (3 Meter) Length

b. 80 - 85 shore A durometer

4.   Catch basins: # 2900 fabricated of polymer concrete

a. Width:  6" OD, 4" ID

b. Depth:  22.1"

c. Length: 1/2 meter (19.7")

d. Grate locking slots: blind, vibration dampening, thermoplastic.

5.	Accessories:

a. Grate Locking devices.

## PART 3: EXECUTION

3.03 INSTALLATION / QUALITY ASSURANCE

1. Install per the manufacturer’s instructions and as indicated on the plans; or by a factory trained and authorized installer.

END OF SECTION