Section 33 4460 Precast Channel Drain and Artificial Turf Anchoring System

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

Section 33 4000 - Storm Drainage Utilities

SYSTEM DESCRIPTION

System of one meter (1m) long channels of polymer concrete with interchangeable plastic edge inserts, drainage and anchoring covers, and accessories. Assembly number:  SE424

Part 2: Products

2.01 PRE-FABRICATED CHANNEL DRAIN

ACCEPTABLE MANUFACTURERS:

Surface drain and artificial turf anchoring system; as manufactured by:

SPORTSEDGE® PRO 424 as manufactured by: SportsEdge P.O. Box 837 / 259 Murdock Rd. Troutman, NC 28166 Telephone: 800-334-6057  Fax: 704-528-0179 www.sportsedge.com

B. COMPONENTS

- Channel: #191SE fabricated of polymer concrete, 150 mm (6 in) wide, 100 mm (4 in) ID and 220 mm (8.8 in) deep with radius non-sloping bottom having the following attributes:

  - Length: 1000 mm (39.2 in).

  - Depth: 9” Minimum

  - Anchoring ribs: full length.

  - Grate locking slots: blind, vibration dampening, thermoplastic.

  - Interlocking ends.

- Grates / Extension Bracket: Fabricated of galvanized steel.

  - #424: Transverse drainage openings and longitudinal carpet retention slot, full length, 1M.

  - #431 Extension bracket, 1M.

- Edges: FPVC, UV Stable, self-retaining, #970L if required.

- Catch basins: #2900SE fabricated of polymer concrete

  - Width:  6", 4" ID

  - Depth:  22.1"

  - Length: 1/2 meter (19.7")

  - Grate locking slots: blind, vibration dampening, thermoplastic.

- Accessories:

  - Grate locking devices.

  - Infill Turf Extension Bracket

## PART 3: EXECUTION

3.03 INSTALLATION / QUALITY ASSURANCE

1. Install per the manufacturer’s instructions and as indicated on the plans; or by a factory trained and authorized installer.

END OF SECTION