Section 33 4460 Precast Track Perimeter Channel Drain

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

Section 33 4000 - Storm Drainage Utilities

SYSTEM DESCRIPTION

System to be SportsEdge PRO-S Channel Drain System of one meter (1m) long drain channels manufactured of Polymer Concrete with removable grates, grate locking devices, and accessories. Assembly number SE2192.

PART 2: PRODUCTS

2.01 PRE-FABRICATED CHANNEL DRAIN

ACCEPTABLE MANUFACTURERS

SURFACE DRAIN: Base; as manufactured by: SportsEdge P.O. Box 837 259 Murdock Rd. Troutman, NC 28166 Telephone: 800-334-6057 Fax: 704-528-0179 Email: info@sportsedge.com

## B. COMPONENTS

1. Channel: # 2192SE fabricated of polymer concrete, 6 inches wide, 4 inch ID and 8.75” deep minimum with radius non-sloping bottom having the following attributes:

a. Length: 1 meter (39.2 in).

b. Depth: 8.75" minimum O.D.

c. Anchoring ribs: full length.

d. Grate locking slots: blind, vibration dampening, thermoplastic.

e. Interlocking ends.

2. Grates: Fabricated of galvanized steel.

# 420: Full width transverse drainage openings.

3.  Catch basins: # 2900 fabricated of polymer concrete

a. Width:  6" OD, 4" ID

b. Depth:  22.1"

c. Length: 1/2 meter (19.7")

d. Grate locking slots: blind, vibration dampening, thermoplastic.

4.	Accessories:

a. Grate Locking devices.

## PART 3: EXECUTION

3.03 INSTALLATION / QUALITY ASSURANCE

1. Install per the manufacturer’s instructions and as indicated on the plans; or by a factory trained and authorized installer.

END OF SECTION