SECTION 11 6600

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

A. Football goals that rotate about a substantially vertical axis180 degrees to face the opposite direction of the football field when the soccer field is in use. Goals shall be manufactured of aluminum with an 8’ minimum offset, and 20’ minimum length uprights. Assembly number SEF305R.

### PART 2 PRODUCTS

2.01 	ROTATING FOOTBALL GOAL POST

A. Base: Model #SEF305R as manufactured by:

SportsEdge®

P.O. Box 837, 259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Rotating Football goals shall be manufactured of aluminum with an 8’ (96”) offset gooseneck that rotates at the base, and must meet the following criteria as a minimum:

1. Gooseneck: fabricated of .280” thickness 6” SCH40 aluminum pipe, 6.625” OD minimum.

a. Rotatable up to 360 degrees

b. Internal Shaft:

    - shall be solid aluminum

    - 47.5” min. length to provide maximum stability

c. Internal Bearing System:

    - shall have 6” minimum diameter bearings

    - shall have two bearing points 31” apart to provide maximum stability.

    - shall not require lubrication

    - shall be made of dissimilar metals for corrosion resistance

d. Rotation point shall be above finished grade.

e. Shall have two keyed locking points to ensure 180 degree rotation for squareness to the end line.

f. Gooseneck unit must be capable of re-calibration for precise alignment of the crossbar to the end line.

2. Crossbar: fabricated of .280” thickness 6” SCH40 aluminum pipe, (6.625 in) OD         minimum.

a. Length: College, 18’-6” between uprights.

b. Length: High School: 23’-4” between uprights.

3. Uprights: fabricated of aluminum tube, 4.0” OD minimum, Length: 20’-0”

4. Installation package consisting of the following components:

- GROUND SLEEVE (required):  8” SCH40 steel pipe, 5’ long, 10”x10”x.125” Steel Plate.

5. Accessories:

- 48” Handle for rotating the goal post when required.

- 4 ea. - Directional wind flags.

- Assembly bolts, nuts and spring (set) pins.

- Touch up paint.

2.02 SOCCER GOALS

A. BASE: Shall be Model #SE700 by:

SportsEdge®

P.O. Box 837, 259 Murdock Rd.

Troutman, NC  28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Model #SE700 Soccer Goals meeting NCAA and NFHS specifications; shall be all aluminum construction with White powder coated finish and must meet the following criteria as a minimum standard:

1. Frame: 8’H x 24’W x 4’B x 10’D.

  - SPECIFIER SELECT:

    - 4” Square aluminum tubing #SE700S

    - 4” Round aluminum tubing  #SE700R

  - White powder coated finish

- Backstays:

  - 2” O.D. Aluminum Backstays, 0.125” Wall thickness

  - White powder coated finish

- Ground Bar:

  - 2” O.D. Aluminum, 0.125” Wall thickness

  - White powder coated finish

  - Must be adjustable & moveable

- Net Attachments:

  - Crossbar & Uprights: Twist in nylon net clips

  - Backstays & Ground bar” Hoop & Loop strap

5. Nets: #SE755 3mm HTTP Nets, 4.75” square mesh.

6. Ground Anchors:

a. Natural Grass Field Applications: #SE752 16” Steel Spiral Anchor w/ Lanyards & quick clip attachment (Set of 4)

b. Synthetic Turf Field Applications: #SE753 (Set of 4) Heavy Duty Nylon Sand Bags (Holds 40 lbs.)

7. Wheel Kit: (Optional) #SE751 rear mounted backstay wheel attachment (set of 4), 10” flat free polyurethane Marathon® tire with nylon bearing.

2.03 	SOCCER GOAL SAFETY ANCHOR SYSTEM

A. Base: Shall be Model #SEF390 by:

SportsEdge®

P.O. Box 837, 259 Murdock Road

Troutman, NC  28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Install around the base of football goal post gooseneck to securely anchor the ground bar of the soccer goal.

1. Nylon straps

2. Dual aluminum stabilizing hooks and ratchets

a. Stainless steel internal parts

3. Aluminum Frame, 22” x 22” X 12” Depth

4. Weather treated plywood cutouts

2.04 	FOOTBALL GOAL POST PADS

A. Base: Shall be Model #SEF302R by:

SportsEdge®

P.O. Box 837, 259 Murdock Road

Troutman, NC  28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

a. 18” minimum thickness split cylindrical urethane foam core, custom manufactured to fit the rotating goal properly.

b. Vinyl laminated polyester fabric, 18 oz. / square yard minimum weight.

c. Minimum height of (6.0’) 72”.

d. Full length Velcro closure strip.

*Note to specifier; owner to select from the following standard colors:			  		Gold, Black, Red, Forest Green, Kelly Green, Brown, Maroon, Orange, Gray,

Royal, Navy, Purple, Tan, White, Sky Blue and Teal.

C. ACCESSORIES:

a. Custom colors, logo, and lettering are optional.

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended by manufacturer, and as indicated on the drawings.

END OF SECTION