SECTION 02880

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - DESCRIPTION

A. Tennis Windscreen.

### PART 2 PRODUCTS

2.01 TENNIS EQUIPMENT

A. Base; manufacturers and product selections named are provided to establish the minimum standard, and shall be TUFFY® Windscreen as supplied by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

F: 704-528-0179

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

A. WINDSCREEN: Model #SETWS-T

1. Construction: 	1000d X 800d Vipol® Matrix Mesh (18 X 14 ends/inch).

2. Fabric Weight: 	10.0 oz. per square yard.

3. Shading:		78% Shading

4. Tensile Strength:    	360 X 320 pounds

5. Sewn Hems: 	Three-Ply hem with ends and corners sewn finished with two rows of lock-stitched thread.

6: Thread:	High heat bonded polyester with UV inhibitors built into yarn.

7: Grommets: 		#2 brass grommets every 12’’ on all four sides.

8. Seams: 	6’ Screens are solid panel (no seams). 9’ screens are prayer seamed with RF weld and one row of black UV treated lock- stitched  thread at center of screen (4 ½’) with grommets every 12’’.  RF welding takes the place of any reinforcing tapes providing a stronger seam.

9. Colors:	Midnight Green, Black, Navy and Royal Blue. Other colors are available.

10. Logos:	      Available single or multi-color.

11. Warranty:	      Pro-rated 48 months on material and workmanship.

### PART 3 EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended with manufacturer's written directions, and as indicated on the drawings.

END OF SECTION