Section 33 4000 Stormwater Utilities

PART 1: GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 33 3000 – Site Improvements

Section 32 1826 - Synthetic Grass Surfacing

Section 32 1822 – Synthetic Athletic & Sports Surfacing

Section 12 9300 – Site Furnishings

Section 03 3000 – Cast-in-Place Concrete

Section 11 6500 – Athletic & Recreational Equipment

Section 33 4000 - Stormwater Utilities

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- System of two meter (2m) long and one meter (1m) long slot drain channels of polymer concrete and schedule 40 PVC pipe, in-line polymer concrete catch basins, and accessories. Slot width shall not exceed ½” (0.5”) in compliance with ADA requirements. Model #XT-6

PART 2: PRODUCTS

2.01 MANUFACTURERS/SUPPLIERS

A. Manufacturers/Suppliers and product selections named are provided to 			establish the minimum standard and shall comply with the following; Base shall 			be SportsEdge® Slot Drain as manufactured by:

SportsEdge®

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

FX: 704-528-0179

info@sportsedge.com

www.sportsedge.com

## 2.02 SURFACE DRAINAGE

A.  Drain Channel: #XT-6; location as shown on the plans, fabricated of 			polymer concrete, schedule 40 PVC pipe, 6-inch ID, 2-meter (78.38 in) 			lengths for the straight aways,1-meter (39.19 in) lengths for the track 			radius, with radius non-sloping bottom having the following attributes:

1. 12mm (15/32”) maximum width polymer concrete drainage slot 			with interlocking ends, with polymer concrete reinforcing at 	6.25” 			intervals.

2. Overall Depth: 8.125” OD (206mm)

3. Length:2 meter (78.38 in).

4. Joint Coupler: PVC Slip coupler, 3.75” minimum length.

5. Radiused Drain for D-Zone (if required): Length 1M (39.19 in), R=35.5M

B.  Catch basins: #2900XTA fabricated of polymer concrete.

1. Width: 6.19", 4" ID

2. Depth:  22.1"

3. Length: 1/2 meter (19.7")

C. Cover: polymer concrete, length ½ meter (19.7"), with 12mm (15/32”) slot, 	removable.

1. Locking device for cover.

D. Accessories: XT-6PC connector plate, fabricated of polymer concrete.

1. Pro channel drain / XT slot drain Connector, when required.

2. PVC end caps, when required.

## PART 3: EXECUTION

3.03 INSTALLATION / QUALITY ASSURANCE

A. Install per the manufacturer’s instructions and as indicated on the plans; or by a 	factory trained and authorized installer.

END OF SECTION