SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following baseball field equipment:

1. Baseball Foul Poles

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 (31 2300) - Site Preparation

Section 02511 (32 1216) - Asphalt Paving

Section 02790 (32 1823) - Athletic & Recreational Surfaces

Section 03300 (03 3000) - Cast in Place Concrete

Section 11480 (11 6600) - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of baseball field equipment. Verify dimensions by field measurements.

### PART 2 PRODUCTS

2.01 BASEBALL / SOFTBALL PRODUCTS

A. Base; manufacturers and product selections named are provided to establish the minimum standard, and shall be as supplied by:

SportsEdge®

P.O. Box 837, 259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

1. FOUL POLES: 30’ Baseball Foul Pole, Model SE330003 shall be manufactured to meet the following criteria as the minimum standard:

A. Main Upright: Fabricated of 5.5625” O.D. heavy wall steel pipe consisting of lower and upper sections.  Lower section shall incorporate an internal sleeve at the upper end, with the upper section installed over the internal sleeve.

B. Wing Panel: Shall be of a unitized design, consisting of expanded metal fabric on a tubular frame.  Wing Panels are 22’ high, 13" wide, start at 8'-0" above the playing surface, and extend to the top of the pole.  Panels are bolted to the upper pole section, and are secured to each other at their outer edges with splice plates.

C. Finish: The entire baseball foul pole assembly shall be finished in a durable yellow powder-coated finish.

D. Ground Sleeve:  Steel, model #SE00276000

### PART 3 EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended with manufacturer's written directions, and as indicated on the drawings.

END OF SECTION