SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Take Off Board System; Long Jump, NCAA.

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Manufacturers and product selections named are provided to establish the minimum standard and shall comply with NFHS requirements.

Equipment shall be as supplied by:

SportsEdge, P.O. Box 837, 259 Murdock Rd. Troutman, NC 28166

Telephone: 800-334-6057, Fax: 704-528-0179, Email: info@sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with NFHS and NCAA requirements.

B. Long Jump Take-Off Boards:

1. Model # SE4350S, Adjustable Aluminum Take-off Board System consisting of an aluminum tray assembly with a cover, a leveling board, and separate take-off and foul boards.  The tray is to be fabricated from 3/16" thick 6061-T6 aluminum sheet and angle which is securely welded together. The lid is fabricated from a 5/16” thick 6061-T6 aluminum sheet top and 3/16" thick supports. Replaceable synthetic take-off board made of high durability polyurethane to resist warping or splintering.  When the take-off boards are not in use, the lid replaces the board. The contractor is to apply runway surface to the lid to make the lid flush with the runway. Leveling board to be fitted with six (6) adjustable leveling feet and two (2) lateral constraint adjustments to eliminate any motion in the tray and ensure a flush fit for the takeoff board. Takeoff and foul boards are to be fabricated from MDO plywood and sealed and painted with outdoor enamel coating.

3.03 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --