SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Pole Vault Pad

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Manufacturers and product selections named are provided to establish the minimum standard and shall comply with the requirements in NFHS rule book.

Equipment shall be as manufactured by:

SportsEdge, P.O. Box 837, 259 Murdock Rd. Troutman, NC 28166

Telephone: 800-334-6057, Fax: 704-528-0179, Email: info@sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with requirements of the NFHS rule book.

B. Pole Vault Pit Pad:

1. Model # SE654 with 18 oz. Vinyl Ground Cover, Planting Box Safety Collar

#SEPBSC and having the following attributes:

a. Overall Size; 19’-9”’W x 20’-5”D x 28”H

1. (3) Base units: 6'7" x 12'9" x 26"

2. (2) Front Tri-slant units: 8'3" x 7'4" x 26" to 16"

3. (1) Top Pad unit: 19'3" x 19'8" x 2"

b. 18 oz. vinyl coated woven polyester, 390 psi puncture resistance.

c. Handles: Double box “X” stitched

d. Top Pad: UV and spike resistant, 2” foam.

e. Handles: 2” nylon web.

f. Hardware: Nickel snap hooks and stainless “D” rings

g. Ground cover, 18 oz. vinyl (optional)

PART 3: EXECUTION

3.03 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --