SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Pole Vault Box and Lid.

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Manufacturers and product selections named are provided to establish the minimum standard and shall comply with NFHS requirements.

Equipment shall be as supplied by:

SportsEdge, P.O. Box 837, 259 Murdock Rd. Troutman, NC 28166

Telephone: 800-334-6057, Fax: 704-528-0179, Email: info@sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with NFHS, NCAA, and IAAF requirements.

B. Pole Vault Box with Cover:

1. Vault Box Model SE505, fabricated from 100% cast aluminum from #319 aluminum alloy with all sides cast to a thickness of ½ inch (.5”), and a white powder coat finish. Inside corners rounded for unhindered pole rotation. Vault box incorporates an 8 inch [20.3cm] forward lip, sloping downward at 30 degrees. Two outer side wings secure the box in place when placed in concrete.

2. Vault Box Lid Model SE503, manufactured from 1/8” (.125”) steel flooring plate with a medium raised lug pattern to prevent slips.  Lids are finished on both sides with durable white powder coat paint.

PART 3: EXECUTION

3.03 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --