SECTION 02880

PART 1 – GENERAL

1.01	SUMMARY

A. This section may include, but is not limited to the following track & field equipment:

1. Shot Put Toeboard; wood, level concrete pad.

B. Related Sections: Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast in Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data 	     required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of track and field equipment. Verify dimensions by field measurements.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A. Manufacturers and product selections named are provided to establish the minimum standard and shall meet all NFHS requirements for high school use.

Equipment shall be as supplied by:

SportsEdge, P.O. Box 837  259 Murdock Rd. Troutman, NC 28166

Telephone: 800-334-6057  Fax: 704-528-0179  Email: info@sportsedge.com

2.02	TRACK AND FIELD EQUIPMENT

A. General: Provide equipment complying with requirements in NFHS rule book.

B. Shot Put Toeboard:

1. Model # SE360, Wood Toeboard, fabricated from select yellow pine and sealed with two coats of white enamel paint, designed to be used on a level throwing surface with Model #SE373 Powder Coated Steel Shot Circle

PART 3: EXECUTION

3.03 INSTALLATION

A.  All athletic equipment shall be installed as recommended by manufacturer's written directions, and as indicated on the drawings.

-- END OF SECTION --