Section 33 4460 Precast Track Perimeter Channel Drain

PART 1: GENERAL

1.01 RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 03 1500 - Concrete Accessories

Section 22 1420 - Storm Drainage Piping

Section 31 2300 - Excavation and Fill

Section 32 1820 - Athletic & Sports Surfaces

Section 32 1826 - Artificial Grass Surfacing

Section 03 3000 - Cast-in-Place Concrete

Section 33 4000 - Storm Drainage Utilities

SYSTEM DESCRIPTION

System to be SportsEdge PRO Channel Drain System of one meter (1m) long drain channels of polymer concrete, with three meter length pliable “L” trim strips for a safe finished edge at the surface, with no polymer concrete channel edges exposed in the finished installation. Drain system to have removable galvanized steel perforated grates, locking devices, and accessories. Assembly number 410SE.

PART 2: PRODUCTS

2.01 PRE-FABRICATED CHANNEL DRAIN

ACCEPTABLE MANUFACTURERS SURFACE DRAIN:

Base; as manufactured by:

SportsEdge

259 Murdock Rd.

Troutman, NC 28166

PH: 800-334-6057

info@sportsedge.com

www.sportsedge.com

## B. COMPONENTS

1. Channel: # 191SE fabricated of polymer concrete, 6 inches wide, 4 inch ID and 9” O.D. deep minimum with radius non-sloping bottom having the following attributes:

a. Length: 1 meter (39.2 in).

b. Depth: 9" minimum O.D.

c. Anchoring ribs: full length.

d. Grate locking slots: blind, vibration dampening, thermoplastic.

e. Interlocking ends.

2. “L” Edge: shall be pliable FPVC, self-retaining, UV stable.

a. # 970L, 3 Meter Lengths

b. 90 Durometer

3. Grates: Manufactured of Galvanized Steel.

a. Model #410SE: Perforated Galvanized Steel, 0.25” holes maximum

b. Length: 1 meter length.

c. Meeting ADA and Heel Proof requirements

d. 14 Gauge minimum

4.  Catch basins: # SE900 fabricated of polymer concrete

a. Width:  6" OD, 4" ID

b. Depth:  22.1"

c. Length: 1/2 meter (19.7")

d. Grate locking slots: blind, vibration dampening, thermoplastic.

5.	Accessories:

a. Grate Locking devices.

## PART 3: EXECUTION

3.01 INSTALLATION / QUALITY ASSURANCE

1. Install per the manufacturer’s instructions and as indicated on the plans; or by a factory trained and authorized installer.

END OF SECTION